# Rules for Blends and Modifiers

# 1. Blended Material Impact Rules

• For material blends, impacts are calculated using a weighted average based on percentage composition.

Example: An 80% cotton / 20% polyester fabric:

CO₂total = 0.8 × CO₂_cotton + 0.2 × CO₂_polyester

Watertotal = 0.8 × Water_cotton + 0.2 × Water_polyester

Energytotal = 0.8 × Energy_cotton + 0.2 × Energy_polyester

• Each material keeps its own chemistry score and circularity score, averaged by blend percentage.

• Elastane: Add +5% penalty to total CO₂ and chemistry scores for every 5% elastane due to recyclability constraints.

# 2. Positive Modifiers

• Recycled Content (rPET, recycled cotton, recycled nylon)

– Reduce CO₂ and energy impacts by 30–60% depending on material type.

– Apply water reduction up to 90% for recycled cotton.

• Organic Cotton

– Reduce water impact by 60%.

– Reduce chemistry impact score by 40%.

• Certified Lyocell / Tencel

– Reduce chemistry impact by 30%.

# 3. Negative Modifiers

• PU Coatings

– Add +40% to CO₂ impacts.

– Chemistry score reduced by −20 points.

• PVC Coatings

– Add +60% to CO₂ and +50 MJ/kg to energy impacts.

• Hazardous Finishes (DWR, PFAS-class)

– Chemistry score automatically set to the lower of:

(base_score − 50) or 10 (worst case)

• High Elastane (>10%)

– For recyclability, circularity score capped at 30.

# 4. Implementation Logic

• Start with base LCA values from the materials library.

• If the fabric is a blend, compute a weighted average across all impacts.

• Apply positive modifiers (reductions) first.

• Apply negative modifiers (penalties) after reductions.

• Normalize results into scores and feed into Eco-Score model.